11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Date Prepared: 3/14/2011

Material Safety Data Sheet

Section 1: Product and Company Identification

Product Name:          Polyaspartic Clear Coat-Part B

Sold By:                    Brand X Coatings

11803 N Saguaro Blvd. Ste. 11

Fountain Hills, AZ 85268

1(480) 222-9720

FOR CHEMICAL      Spill, Leak, Fire, Exposure,

EMERGENCY          or Accident, Call 24 hours

1(480) 222-9720

Fax Phone:               1(480) 222-9711

Section 2: Composition/Information on Ingredients

Product Identification                                          CAS#           ACGIH TLV          OSHA PEL

Aminofunctional Recactive Resin Mixture         Trade Secret        N/A                            N/A

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 3: Hazards Identification

Emergency Overview:

Routes of Entry:                    Route                 Entry Risk

Inhalation           Possible, unlikely due to low vapor pressure

Ingestion            Possible

Skin Contact       Possible

Eye Contact        Possible

Potential Health Effects: Inhalation          May cause respiratory tract irritation.

Ingestion          Ingestions is not a typical route of industrial exposure. Not 						 expected to be harmful if swallowed.

Skin Contact     Product is a slight skin irritant. May cause swelling and redness.

Eye Contact      Mild eye irritant. Vapors slightly uncomfortable.

Acute Health Hazards:   Acute ingestion is not a typical route of industrial exposure. Not expected to be 				 harmful if swallowed. Inhalation of vapors can cause respiratory tract irritation.

Chronic Health Hazards:  Prolonged contact can cause reddening, swelling, rash, and in some cases skin 			  sensitization.

Medical conditions           Skin and eye sensitization. Depending on individual skin sensitivity, chronic or Generally Aggravated	  prolonged exposure may result in irritation, swelling, and rash.                          by Exposure:

Carcinogenicity:               OSHO: No data    ACGIH: No data   NTP: No data   IARC: No data

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 4: First Aid Measures

Eye Contact:                    Immediately flush with plenty of water for at least 15 minutes. If redness, itching,              			              or a burning sensation develops, seek medical attention.

Skin Contact:                   Remove from skin immediately. Rinse with clean water for 20-30 minutes. Use                			                              soapy water if needed. If redness, itching or a burning sensation develops, seek  			                              medical attention.

Ingestion:                         Do NOT induce vomiting! Dilute with water and seek medical attention 					                            immediately.

Inhalation:                       Move victim to fresh air immediately. Give oxygen and seek medical attention.

Section 5: Fire-Fighting Measures

Flammable Limits:         Not determined

Flash Point, (Method Used):    Approximately 145 C (293 F)

NFPA Hazard Rating:            Health: 2        Hazard scale     Protective Equipment

Flammability: 1        0=LEAST                 A=SAFETY GLASSES

Reactivity: 0        1=SLIGHT                B=SAFETY GLASSES, GLOVES

Other:           2=MODERATE        C=SAFETY GLASSES, GLOVES AND 								               APRON

HMIS Hazard Rating:         Health: 2           3=HIGH

Flammability: 1            4=EXTREME

Reactivity: 0

Protection: B

Extinguishing Media:         Foam/Carbon Dioxide/Dry Chemical/Water Fog

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Special Fire Fighting Procedures:       Remove all ignition sources. Wear self-contained breathing 						                                   apparatus and complete personal protective equipment when 					                                   entering confined areas where potential for exposure to vapors or 				                                                 products exist.

Unusual Fire and Explosion Hazards:     Closed containers may rupture due to build up of pressure when 					                                          exposed to extreme heat.

Section 6: Accidental Release Measures

Accidental Release Measures:   Standard hydrocarbon spill procedures apply to this product.

Section 7: Handling and Storage

Handling & Storage Precautions:  Prevent all skin and eye contact. Avoid breathing vapors. Re-seal partially used containers. Wash with soap and water before eating or drinking. Protect from moisture contamination. Exothermic generation of carbon dioxide may cause dangerous pressure. Keep away from all ignitable sources as well as extreme heat. Do not expose to excessive moisture.

Section 8: Exposure Control/Personal Protection

Ventilation:  Adequate ventilation required. Local exhaust may be required in some areas. Special exhausting generally not required. Mechanical exhaust usually adequate.

Respiratory Protection:  Respiratory masks should be worn at all times when adequate ventilation does not exist. A NIOSH/MSHA respirator is acceptable.

Eye Protection:  Chemical tight goggles; full face shield if splashing is possible.

Skin Protection:  Coveralls and impervious foot covering is recommended.

Other Protective Clothing or equipment:  Use impervious gloves, neoprene or rubber.

Work/Hygienic Practices:  Good air flow in working area. Eyewash station and safety shower should be available. Gloves and respiratory equipment should be worn at all times.

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 9: Physical and Chemical Properties

Appearance:			Amber

Odor:				Slight

Physical State:			Liquid

ph as Supplied:		Not applicable

Boiling Point:			185 C (365 F) @ 1013 mbar

Melting Point:			Not determined

Pour Point:			Approximately -15C (5)

Vapor Pressure (mmHg):	0.000014 mmHG @ 20 C (68 F)

Vapor Density (air =1):              1.06 g/cm3 @ 20F (68 F)

Specific Gravity (Water=1):	1.056 @ 77 F

Volatile Organic Compounds (VOC):	With water: None     Without Water: None

Solubility in Water:		Insoluble

Viscosity: 			Approximately 1,450 mPas @ 25 C (77 F)

Section 10: Stability and Reactivity

Stability: Stable

Conditions to Avoid (Stability): Avoid excessive heat, open flame, sparks and strong oxidizing agents. Protect from atmospheric moisture. Replace outage with inert dry nitrogen.

Incompatibility (Materials to Avoid): Avoid strong oxidizers, acids.

Hazardous Decomposition or Byproducts: Carbon dioxide, carbon monoxide, nitrogen oxides.

Hazardous Polymerization: Will not occur.

Conditions to Avoid (Polymerization): Avoid incompatible reactants, especially strong bases, water or temperatures over 160 degrees centigrade.

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 11: Toxicological Information

Toxicological Information (Acute Toxicity): No data

Section 12: Ecological Information

Ecological Information:  No Data

Section 13: Disposal Considerations

Waste Disposal Method: Dispose of according to current local, state, and federal regulations.

RCRA Hazard Class: Non-Regulated

Section 14: Transport Information

U.S. Department of Transportation

Proper Shipping Name: Polyol Resin

Hazard Class: Non-Regulated, NOI

Water Transportation: Hazard Class: Non-regulated, NOI

Air Transportation: Hazard Class: Non-Regulated, NOI

Section 15: Other Information

Disclaimer: This information is formulated for use by elements of the Department of Defense. The United States of America in no manner whatsoever expressly or implied warrants, states, or intends said information to have any application, use or viability  by or to any person or persons outside the Department of Defense nor any person or persons contracting with any instrumentality of the United States of America and disclaims all liability for such use. Any person utilizing this instruction who is not a military or civilian employee of the United States of America should seek competent professional advice to verify and assume responsibility for the suitability of this information to their particular situation regardless of similarity to a corresponding Department of Defense or other government situation.